Technische Hochschule Ulm

Digital Twin of Kuka KR3

User Guide

Ibrahim Almohamed, Ahmed 12.08.2024

Contents

1.	Version and Control	2

2	Setup robot HMI mirroring and shared data driver	3

2.1	USB configuration  and robot setup	3

2.2	Windows IPV4 configuration and VNC viewer	21

# Version and Control

| Version | Name of Editor | Notes | Date |
| --- | --- | --- | --- |
| 1.0.0 | Ahmed Ibrahim Almohamed | n/a | 22.09.2024 |

# Setup robot HMI mirroring

## USB configuration  and robot setup

Grab a usb (memory size must be < 4GB) , then on your windows OS do the following :

- format the usb into NTFS file format.

- go to https://www.tightvnc.com/download.php  and install BOTH of the vnc installers.

- Then copy the 32bit installer  on the usb

- stick the usb on the usb ports of the robot.

- after that on the SmartPad (you have to start the robot) switch the users to admin (the password is :  kuka).

- after that the usb will show on the HMI  with the vnc downloader.

- After that click the robot icon on top left

- go to service -> Minimize HMI then (image) the windows that is running on the KRC4

- shows up (desktop)(image)

- after that open the file explorer and go to your usb driver and run the downloader.

- just run the downloader with all the default settings .

- after finishing go back to the HMI (Image)

- click again on the robot icon and select Network configuration and wait for some seconds till the configuration start.

- go the Advanced,

- you can see the KLI configuration , this is the connection we are using on this project . now first write down the ip of the KLI (image) .

- then go to NAT button (image) -> add port , change the port number to 5900  and the permitted protocols to tcp/udp and click save (image) .

- After that stay as Admin , click on the robot icon -> shutdown , Reload files (check) and Reboot control PC (image)

Now we are done with the robot and from now on we shall only use the mirroring viewer of the TightVNC .

## Windows IPV4 configuration and VNC viewer

- After setting up the robot, connect the robot to the PC via a LAN (Ethernet) cable at the KLI port (Fig.).

- Then go to Windows Control Panel -> Network and Internet -> Change adapter settings -> Right-click on your Ethernet card -> Properties -> IPv4 and select Properties , change the ip address to something near the one on the robot (by me on the robot it was 172.31.1.197) so i set the ip on windows to : 172.31.1.100. click ok and close the window (image).

- now open a cmd prombt and type : ping <ip_of_the_robot> by me it is ping 172.31.1.197 (image).

if you revice a response then every thing is working right .

now we already installed the TightVNC for windows (64 bit) install it and run the viewer(TightVNCViewer)(image).

- input the ip and connect , then the HMI of the robot starts on your PC.

(if the HMI is not fully viewed (some parts are not showing ) then go admin -> minize HMI -> right click on the robot desktop  -> display setting -> change orientation to landscape).

HINT : if when starting the viewer session , the kuka asks you to log in (like windows log in screen ), then please use the following creds : user name : KukaUser , password : 68kuka1secpw59

# Shared Data driver

we are not using the usb after this guide we share the data simply between the host PC and the Kuka robot .

first start the robot and on the pc start the VNC viewer .

Log in as admin and minimize the HMI .

- Open the file explorer and right click on the KUKA_DATA(D:) driver  and select properties(image).

- go to sharing ->Advanced sharing - Permissions -> check full control(image).

- after that on your windows PC (host ) open the file explorer and in then type \\<ip_of_the_robot>(image). and that’s it .

now if you want to always have the driver on your pc , on your file explorer right click network , connect a network and on the network name insert : \\<ip_of_robot>\d.  this then create a driver that connects to the robot.